AVCO-BUFFER DLT HC

ACID DONOR AND pH REGULATOR FOR DYEING POLYAMIDE AND WOOL

AVCO-BUFFER DLT HC is an highly concentrated acid donor which generates, very slowly, acidity upon heating up during the dyeing process. The pH of  1 g/l AVCO-BUFFER DLT HC is 6.5-7.0, and when heated to the boil the pH  decreases to pH 4- 5 .

AVCO-BUFFER DLT HC is mainly recommended for dyeing pale to dark shades.

The rate of dissociation is slow in cold solutions, and increases while heating up. This slow dissociation process is useful for dyeing wool and polyamide fibers, as the dyes are exhausted quite evenly and produce levelled dyed yarns and fabrics.

SPECIFICATIONS:

| Appearance | Yellowish or colorless liquid |
| --- | --- |
| Chemical nature | Blend of organic esters |
| Solubility | Miscible with water. Solutions are not stable and start to hydrolize |
| pH (0.1 %) | 6.5 – 7.0 (fresh solution) |
| Ionic type | Non-ionic |
| Compatibility | Is not comopatible with strong alkalis. Hydrolizes in aqueous solutions. |
| Storage | Stored in a closed container in a cool place. Do not store in direct sun |

PROPERTIES & USES:

1.	AVCO-BUFFER DLT HC is an acid donor which evenly and slowly hydrolizes in hot aqueous solution to produce an organic acid.

2.	During the hydrolysis process the organic acid produced is lowering the pH of the solution down to 4 - 5 according to the concentration in the liquor.

3.	The rate of the dissociation is slow in cold solutions and increases when the temperature is raised.

4.	This manner of pH control makes it useful for dyeing acid and metal complex dyes. As the exhaution rate of acid dyes is pH dependent, the slow lowering of the pH in a dyeing solution containing AVCO-BUFFER DLT HC, ensures more levelled dyeing, due to slower exhaution rate of the dyestuffs onto the fibres.

5.	AVCO-BUFFER DLT HC is recommended for even dyeing of polyamide, wool and blends of these fibres.

- When using AVCO-BUFFER DLT HC, the initial temperature of the dyeing can be increased, and the rate of heating up may be faster, thus reducing dyeing time.

- AVCO-BUFFER DLT HC does not influence any of the fastness properties of the dyed fibres.

APPLICATION:

Following are some recommended dyeing procedures. The dosages, temperatures and procedures are suggestions which may vary according to the type of dyestuffs used and properties of fibres to be dyed.

Do not prepare water solutions of the product before use. Add the product as is to the side tank and dose it to the dyeing machine

1.	Dyeing wool

Use the following recipe:

AVCO-LEV WLS	0.5 - 2.0 %

Ammonium Sulphate	1.0 - 3.0 g/l

AVCO-BUFFER DLT HC	0.3 - 1.0 g/l

Dyestuffs	X %

Start dyeing at 30°C, raise the temperature to the boil in about 45 min.

Dye at the boil for 60 - 90 min. depending on the depth of the shade.

2.	Dyeing wool with an anti-shrinking treatment

Use the following recipe:

AVCO-LEV LWS	0.5 - 2.0 %

ammonia sol. (25%)	1.0 - 2.0 g/l (adjust to pH 8-9).

AVCO-BUFFER DLT HC	0.5 - 1.0 g/l

Dyestuffs	X%

Start dyeing at 30°C, raise the temperature to 70°C at 1.0° C/min. Hold at 70°C for

10 min. Continue heating at 1° C/min. to the boil. Dye at the boil for 60-90 min.

- Dyeing polyamide/wool blends

Use the following recipe:

AVCO-LON APL	0.5 – 2.0 %

AVCO-BUFFER DLT HC	0.5 – 1.5 g/l

AVCO-FIX N40                         1.0 – 2.0 g/l (to prevent rapid striking of dyes onto

The PA fibre).

Dyestuffs	X %

Start dyeing at 30°C, raise the temperature to 60°C at 1.5° C/min. Hold for 15 min.

Continue heating at 1°C/min. to the boil. Dye at the boil for 60 min.

4.	Dyeing polyamide

Use the following recipe:

AVCO-LON APL	0.5 - 2.0 %

AVCO-BUFFER DLT HC	0.5 - 1.5 g/l

Dyestuffs	X

Start dyeing at 40-50°C, raise the temperature to the boil at 0.5-1.5°C/min. Dye at the boil for 60 min. Cool down to 70°C and drop the bath. Rinse with cold water.

# Dosage of AVCO-BUFFER DLT HC

LIGHT SHADES	0.3 – 0.5 g/l

MEDIUM SHADES	0.5 – 0.8 g/l

DARK SHADES	0.8 – 1.5 g/l